# July 21, 2026 — Python ipaddress for Network Automation

Daily Python Learning for Network Engineers  •  Lesson 01

| Time 35–45 minutes | Level Beginner → Practical | Outcome Validate and analyze IPs |
| --- | --- | --- |

# Why this matters

Network automation frequently receives IP addresses and prefixes from YAML files, APIs, device output, or user input. Python’s built-in ipaddress module converts those strings into reliable objects, catches invalid input, and performs subnet math without manual binary calculations.

# Learning objectives

Create IPv4/IPv6 address, network, and interface objects.

Find network address, broadcast address, usable hosts, and prefix length.

Check whether an address belongs to a subnet.

Classify addresses as private, global, loopback, multicast, or link-local.

Apply these checks in a reusable network-validation script.

# 1. The three core object types

| Object | Example | Use |
| --- | --- | --- |
| ip_address | 192.0.2.10 | One IPv4 or IPv6 address |
| ip_network | 192.0.2.0/24 | A subnet/prefix and its range |
| ip_interface | 192.0.2.10/24 | A host address plus its connected subnet |

from ipaddress import ip_address, ip_network, ip_interface

ip = ip_address("10.20.30.40")
subnet = ip_network("10.20.30.0/24")
interface = ip_interface("10.20.30.40/24")

print(ip)                 # 10.20.30.40
print(subnet)             # 10.20.30.0/24
print(interface.network)  # 10.20.30.0/24

# 2. Inspect a subnet

from ipaddress import ip_network

net = ip_network("192.168.50.0/29")
print(net.network_address)    # 192.168.50.0
print(net.broadcast_address)  # 192.168.50.7
print(net.netmask)            # 255.255.255.248
print(net.num_addresses)      # 8
print(list(net.hosts()))      # .1 through .6

A /29 contains 8 total addresses. For traditional IPv4 subnets, the network and broadcast addresses are not assigned to hosts, leaving 6 usable host addresses.

# 3. Test subnet membership

from ipaddress import ip_address, ip_network

mgmt_net = ip_network("10.10.100.0/24")
device_ip = ip_address("10.10.100.25")

if device_ip in mgmt_net:
    print("Valid management address")
else:
    print("Address is outside the management subnet")

# 4. Validate inventory data safely

from ipaddress import ip_address, AddressValueError

def validate_device_ip(value):
    try:
        address = ip_address(value)
        return {"valid": True, "version": address.version,
                "private": address.is_private}
    except AddressValueError as error:
        return {"valid": False, "error": str(error)}

for value in ["10.1.1.10", "2001:db8::10", "10.999.1.1"]:
    print(value, validate_device_ip(value))

Network-engineering use case: run this validation before connecting with Netmiko/NAPALM or before pushing a generated configuration. Invalid inventory data is rejected early instead of causing a confusing connection failure.

# 5. Practical mini-project: audit device IPs

from ipaddress import ip_address, ip_network

allowed = ip_network("10.20.0.0/16")
devices = {
    "rwa01": "10.20.1.11",
    "ewa01": "10.20.2.12",
    "lab-sw1": "172.16.5.20",
}

for name, value in devices.items():
    ip = ip_address(value)
    status = "PASS" if ip in allowed else "FAIL"
    print(f"{status:4}  {name:8}  {ip}")

Expected result: rwa01 and ewa01 pass; lab-sw1 fails because 172.16.5.20 is outside 10.20.0.0/16.

# Practice exercises

Create 172.16.40.0/27 and print its network address, broadcast address, subnet mask, and total address count.

Check whether 172.16.40.31 and 172.16.40.32 belong to 172.16.40.0/27. Explain the result.

Split 10.10.0.0/24 into /26 subnets using net.subnets(new_prefix=26). Print each subnet.

Extend the inventory audit so malformed IP addresses produce ERROR without stopping the program.

Bonus: accept both IPv4 and IPv6 addresses and print each address family.

# Answer check

Exercise 1: network 172.16.40.0, broadcast 172.16.40.31, mask 255.255.255.224, 32 total addresses. Exercise 2: .31 is inside the prefix but is its broadcast address; .32 belongs to the next /27. Exercise 3 produces 10.10.0.0/26, 10.10.0.64/26, 10.10.0.128/26, and 10.10.0.192/26.

# Key takeaways

Use ip_address for a single address, ip_network for a prefix, and ip_interface for a host/prefix pair.

Use strict=False only when you intentionally want host bits normalized into a network prefix.

Validate inventory input before opening device sessions or generating configuration.

Prefer built-in subnet operations over hand-written string or binary logic.

Next suggested topic: Reading YAML device inventories and validating required fields.